|  | MINISTRY OF EDUCATION AND TRAINING |
| --- | --- |

# FPT UNIVERSITY

Capstone Project Document

DESIGN AND CONSTRUCTION SUN DRYING WET CLOTHES SYSTEM

| Group 2 |  |
| --- | --- |
| Group members | Hoàng Phi Long – SE62021 Nguyễn Đình Phong – SE61968 Trịnh Bình – SE61780 |
| Supervisor | Nguyễn Đức Lợi |
| Ext. Supervisor | N/A |
| Capstone Project code | DCDCS |

Ho Chi Minh City, June 26th 2018

# Table of Contents

Table of Contents	2

List of Tables	4

List of Figures	5

Definitions, Acronyms and Abbreviations	6

A.	Introduction	7

1.	Project Information	7

2.	Introduction	7

3.	Current Situation	7

4.	Problem Definition	8

5.	Proposed Solution	8

5.1	Feature Functions	8

5.2	Advantages and Disadvantages	9

6.	Functional Requirements	9

7.	Role and Responsibility	10

B.	Software Project Management Plan	10

1.	Problem Definition	10

1.1	Name of this Capstone project	10

1.2	Problem Abstract	11

1.3	Project Overview	11

2.	Project Organization	13

2.1	Software Process Model	13

2.2	Roles and Responsibility	14

2.3	Tools and Techniques	15

3.	Project Management Plan	16

3.1	System Development Life-cycle	16

3.2	Plan Detail	18

C.	Software & Hardware Requirement Specification	22

1.	User Requirement Specification	22

2.	System Requirement Specification	22

2.1	External Interface Requirement	22

2.2	System Overview Usecase	24

3.	Hardware Requirement Specification	26

3.1	Hardware Interface	26

4.	Conceptual Diagram	27

D.	Software & Hardware Design Description	28

1.	Design Overview	28

2.	System Architectural Design	29

2.1 	API Web Server Architectural Design	29

2.2	Android Application Architectural Design	32

2.3	Hardware System Architecture	34

3.	Component Diagram	36

4.	Detailed Description	38

3.3	Class Diagram	38

3.4	Interaction Diagram	45

4.	Database Design	50

4.1	Entity Relational Database (ERD)	50

4.2	Data Dictionary	50

5.	Algorithms	51

5.1	System Control	51

E.	Task Sheet	52

F.	Appendix	60

# List of Tables

Table 1: General Roles and Responsibilities of Member	10

Table 2: Hardware development environment requirement for DCDCS System	12

Table 3: Software development environment requirement for DCDCS System	13

Table 4: Roles and responsibilities	15

Table 5: Tools and techniques	15

Table 6: Project task planning	17

Table 7: Plain Detail - Requirement Analysis	18

Table 8: Plain Detail - Design	19

Table 9: Plain Detail – Implementation	20

Table 10: Plain Detail –Testing	20

Table 11: Plain Detail –Maintenance	21

Table 12: Data dictionary for conceptual diagram	27

Table 13: Component diagram dictionary	37

Table 14: API Web server class diagram dictionary	41

Table 15: Hardware controller class diagram dictionary	44

Table 16: Entity diagram data dictionary	50

Table 17: Tasksheet	59

# List of Figures

Figure 1: Waterfall methodology	14

Figure 2: Hardware system overview usecase diagram	24

Figure 3: Android application overview usecase diagram	25

Figure 4: System block diagram	26

Figure 51: Conceptual diagram	27

Figure 6: System overview architecture	29

Figure 7: API Web server architecture	30

Figure 8: Android application internal architecture	32

Figure 9: Hardware system architecture	34

Figure 10: Component diagram	36

Figure 11: API Web Server Class Diagram Part 1	38

Figure 12: API Web Server Class Diagram Part 1	38

Figure 13: API Web Server Class Diagram Part 2	39

Figure 14: API Web Server Class Diagram Part 3	40

Figure 15: Hardware system controller class diagram part 1	42

Figure 16: Hardware system controller class diagram part 2	43

Figure 17: Control system with android app sequence diagram	45

Figure 18: Update system information sequence diagram	46

Figure 19: Control DC activity diagarm	47

Figure 20: Control Dryer activity diagram	48

Figure 21: Auto control activity diagram	49

Figure 22: Entity Relational Database	50

Figure 23: System Control overview flowchart	52

# Definitions, Acronyms and Abbreviations

| Name | Definition |
| --- | --- |
| DCDCS | Design and Construction sun Drying wet Clothes System |
| RF | Radio frequency |
| HTTP | Hypertext transfer protocol |
| I2C | Inter-Integrated Circuit |
| UART | Universal asynchronous receiver-transmitter |
| DIY | Do it yourself |
| REST | Representational state transfer |
| API | Application programming interface |
| GPIO | General-purpose input/output |
| I/O | Input/Output |

# Introduction

## Project Information

- Project name: DESIGN AND CONSTRUCTION SUN DRYING WET CLOTHES SYSTEM

- Project Code: DCDCS

- Product Type: Embedded Device, Android Application, API Web Server

- Start Date: 14/06/2018

- End Date: 31/08/2018

## Introduction

An automatic clothes drying system, which uses rain sensor to detect rain and ESP8266 for communications between Android application and embedded device, was developed to allow consumers to effectively manage their chores. This document will explain the foundations and the processes of this innovative system.

Furthermore, this document will describe our working process in 4 months includes our perspective in the system, component designs and detailed core workflows. We hope the system will help resolve some aspects of the problem that the current face recognition systems are facing today.

## Current Situation

Vietnam is a tropical country with a long rainy season accounting for almost half of the year. Vietnamese have a habit and are also much more used to drying their clothes naturally under the sun rather than using the dryer. This habit always pose a problem of inefficient clothes drying process during the rainy months. That is, Vietnamese are constantly worried about not being able to collect their clothes which leaves the clothes potentially getting wet under the rains. Thus, it is necessary to develop a system that helps people to manage their laundry chores better. There has been a few solutions given such as the “Smart Clothesline Rigs”.  However, this system is relatively expensive and ineffective. At 13.000.000 VND, the “Smart Clothesline Rigs” is a controllable system with UV light, build-in dryer and remote control (only works within 30 meters).  Nevertheless, the system does not solve the core problem of allowing the automation of clothes collecting. Thus, the Sun Drying Wet Clothes System, which integrates the automatic clothes collecting function, would better suit the needs of Vietnamese.

## Problem Definition

Advantage of existing system on the market

- UV disinfection

- Built-in dryer

- Strong structure which can lift up to 25kg of clothes

- Drawbacks of existing system on the market

- High production costs which lead to relatively unaffordable selling prices

- Hard to extend

- Automation fully dependent on electricity

- Cannot automatically collecting clothes

## Proposed Solution

Our proposed solution is to design and construct an automatic clothes drying system called DCDCS to solve missing feature of the current “Smart Clothesline Rigs”. Our system will allow the automation of laundry-colleting in the case of rains. Our system will also be competitively priced, easier installation, more compact and mobile, and extendable compare to the existing system.

DCDCS system includes a mobile app and an embedded device with following functions:

### Feature Functions

- Mobile App:

  - Control the system through wireless

  - Check weather information

  - Check system status

- Embedded Device:

  - Check system status

  - Control system through hard buttons

### Advantages and Disadvantages

- Advantages:

  - Low costs which allow more affordable prices

  - Fast rain detection

  - Can control using mobile app

  - Use solar energy and store extra energy as battery for use under adverse conditions

- Disadvantages:

  - Cannot detect whether the clothes is dry or not

  - Cannot detect whether rain is over or not

## Functional Requirements

Functional requirements of the system are listed as below:

- Embedded system component:

  - RESTful API communication through wireless

- Use Arduino Mega 2560 as a central circuit unit

- Show information about the system

- Time

- Temperature

- Humidity

- Control dryer

- Control clothesline

- Power supply component:

- Power supply operates for the entire system

- Distributed voltage 5V and 12V

- Auto charging

- Storing energy

- User component:

- Control the system from Android application through wireless

- Turning on/off build-in dryer

- Set timer for dryer

- Control the clothesline

- Check system status and weather

- Edit user information

- Name

- Address

- Mobile phone

- Etc

- Mobile Application component:

- Communicate with system through wireless and by REST API

- Show information about the system

- Time

- Temperature

- Humidity

- Weather (Rain or not)

- System status

## Role and Responsibility

| No | Full name | Role | Position | Contact |
| --- | --- | --- | --- | --- |
| 1 | Nguyễn Đức Lợi | Project Manager | Supervisor | loinnd@fpt.edu.vn |
| 2 | Hoàng Phi Long | Developer | Leader | longhpse62021@fpt.edu.vn |
| 3 | Nguyễn Đình Phong | Developer | Member | phongndse@fpt.edu.vn |
| 4 | Trịnh Bình | Developer | Member | binhtse@fpt.edu.vn |

Table 1: General Roles and Responsibilities of Member

# Software Project Management Plan

## Problem Definition

### Name of this Capstone project

- Official name: Design and construction sun drying wet clothes system

- Vietnamese name: Thiết kế và xây dựng hệ thống phơi đồ tự động

- Abbreviation: DCDCS

### Problem Abstract

Vietnamese have long working hours which means they spend time at evening and night to do their chores. The chores include washing and drying clothes. However, Vietnam also has long rainy season which indicate a persistent problem of inefficient clothes drying process.

### Project Overview

#### Current Situation

Below are the problems encountered in the project:

- Lack of robust statistic and mathematical knowledge: Our system currently cannot detect when the clothes are dry or when the rain stop for auto collecting clothes or continue drying wet clothes. To do so, it requires mathematics model called Hidden Markov Models. However, due to the lack of knowledge in statistics and linear algebra; we are unable to implement this function.

- Lack of telecommunication knowledge: While using ESP8266, we found that there would be interferences during transmission. Nonetheless, we were experiencing difficulties in detecting problems due to our lack of telecommunication knowledge.

#### The Proposed System

According to the technology researches, we found that the simple rain sensor and ESP8266 Wi-Fi module is capable in solving the problem. We can use rain sensor detect raining and ESP8266 for wireless communication.

We assign task responsibility vertically to make sure if any member in this project fail in our team, harm would be minimized for the project.

We also build a mobile application for real-time demonstration.

#### The Boundaries of the system

Our system provides these functions:

- Automatically control clothesline at nighttime and at raining periods

- Dryer system so that user can dry their clothes on rainy days

- Control system via RF Remote control

- Control system via Button on the system

- Check system status and control system via Mobile application

#### Future plans

- Implement Hidden Markov Models (HMM) for rain forecasting

- Implement the ability to detect when the rain stop using HMM

- Build a website for user to check their account information and control the system along with mobile application

- Build a system that can detect whenever the clothes is dry or wet

#### Development Environments

##### Hardware Development Environment Requirement

For CCU clothes drying system

| Component | Hardware |
| --- | --- |
| Mainboard | Arduino Mega 2560 |
| Communication | Wire and cable |
| Devices | - Module real-time clock DS1307 - Rain sensor - Humidity and Temperature sensor DHT11 - Light sensor BH1750 - DC Motor - Nokia 5110 LCD - 4x4 Matrix keypad - Limit switches - Solar Panel - Battery - … |
| Power source | 5V – 12V |
| Android Device | Any android mobile phone has 3G/4G or Wi-Fi connection |

Table 2: Hardware development environment requirement for DCDCS System

##### Software Development Environment Requirement

| Software | Name / Version |
| --- | --- |
| Operating System | Windows 7 or above |
| Environment/Run-time | Adruino Mega 2560 NodeJS |
| Modeling tool | Draw.io for UML Proteus 8 for PCB Board |
| IDE | Visual Studio Code Arduino IDE |
| DBMS | MongoDB |
| Source control | Git-scm and Github |
| Communication tools | Facebook Messenger Gmail |

Table 3: Software development environment requirement for DCDCS System

## Project Organization

### Software Process Model

This project is developed using modified waterfall model. We apply modified waterfall model because it suitable with current situation in our team. We choose this model because of the following reasons:

- This project is 4-months long due to the FPT University Capstone Project timeline, which can be consider a short project.

- Based on researches and current clarified clothes drying system, the requirements of this project are stable, clear, fixed and well-understood by all team members.

- The Modified Waterfall Model involves verification and validation between the phases, so any deviations can be corrected immediately, providing the customer satisfaction, so this is preferred.

Figure 1: Waterfall methodology

### Roles and Responsibility

| No | Fullname | Role | Responsibilities |
| --- | --- | --- | --- |
| 1 | Nguyễn Đức Lợi | Supervisor, Project Manager | - Specify user requirement - Advisor for ideas and solutions - Give out techniques and business analysis support |
| 2 | Hoàng Phi Long | Team leader, developer, tester | - Managing process - Dividing tasks for team member - Create test plan - Clarifying requirements - Coding - Testing - Verify document - Managing budget - Database design |
| 3 | Nguyễn Đình Phong | Team member, developer, tester | - Create test plan - Database design - Clarifying requirements - Prepare document - Coding - Testing - GUI Design |
| 4 | Trịnh Bình | Team member, developer, tester | - Create test plan - GUI Design - Database design - Clarifying requirements - Prepare document - Coding - Testing |

Table 4: Roles and responsibilities

### Tools and Techniques

| Tools |  |
| --- | --- |
| Developing tools | Visual Studio Code Arduino IDE |
| Database system management | MongoDB |
| Source Control | Git-scm and Github |
| Models and Diagrams tool | Draw.io |
| Techniques |  |
| Embedded System | C/C++ , Arduino SDK |
| API Web Server System | ExpressJS & NodeJS |
| Mobile Application | React Native, Javascript |

Table 5: Tools and techniques

## Project Management Plan

### System Development Life-cycle

Below are all the major tasks that need to be performed sequentially during the development of the system.

| Phase | Description | Deliverables | Resource needed | Dependencies and Constraints | Risks |
| --- | --- | --- | --- | --- | --- |
| Requirement Analysis | - Identify and clarify main functions. - Prepare task plan. - Research mechanics of collecting clothes system - Research solar energy circuit | - Report No. 1 Introduction. - Project Management Plan - Task sheet - Prototypes | 14 man-days | N/A | - Missing requirement. - Unclear project’s scope. - Lack of member share of understand. |
| Design | - Identify hardware and software requirements. - Decide software architecture. - GUI design using top-down break down. - Design database. | - Report No. 2 Software Project Management Plan. - Report No. 3 Software Requirement Specification. - Report No. 4 Software Design Description. | 20 man-days | Depend on “Requirement Analysis”. | - Misunderstood or unclear system’s requirement. - Lack of practical experience leading to unreasonable design. |
| Implementation | - Collect temperature, humidity datasets. - Build hardware system - Implement embedded software system - Implement Android GUI. - Build REST API | - Demonstration application. - Report No.5 System Implementation & Test. | 50 man-days | Depend on “Design”. | - Lack of practical experience and knowledge. - Human mistake. - Broken hardwares due to wrong implementation - Interference signal while ESP8266 communicate with Http Protocal |
| Testing | - Prepare test plan and test case. - Test all functions and results. | Report No.5 System Implementation & Test. | 20 man-days | Depend on “Implementation”. | - Lack of experience. - Not enough time for performing test. - Missing bugs. - Human resource. |
| Maintenance | - Deploy the system. - Create the user’s manuals. | Report No.6 Software User’s Manual. | 10 man-days | Depend on “Testing”. | - Lack of experience and knowledge. - Human mistake. - User’s manual may be difficult for user to understand and confuse. |

Table 6: Project task planning

### Plan Detail

#### Phrase 1: Requirement Analysis

| Task | Description | Author |
| --- | --- | --- |
| 1. Research mechanics of collecting clothes system | - Research on current systems, their strengths and weakness. | Hoàng Phi Long Nguyễn Đình Phong |
| 1. Research solar energy | - Research on current systems, their strengths and weakness. - Research how to convert solar to electricity and charge into batter | Nguyễn Đình Phong Trịnh Bình |
| 3. Identify and clarify main functions | Define main and needed functions the system must include. | Hoàng Phi Long Nguyễn Đình Phong Trịnh Bình |
| 4. Create system introduction | Complete Introduction Report. | Hoàng Phi Long |
| 5. Software Project Management Plan | Prepare Project Management Plan. | Hoàng Phi Long |
| 6. Prototype | Build a prototype of system and mobile application. | Nguyễn Đình Phong Trịnh Bình |
| 7. SRS | Create SRS document. | Hoàng Phi Long Nguyễn Đình Phong Trịnh Bình |

Table 7: Plain Detail - Requirement Analysis

#### Phrase 2: Design

| Task | Description | Author |
| --- | --- | --- |
| 1. Identify hardware and software detail design | Find out the suitable hardware and software for the system, as well as its minimum and recommended requirements. | Hoàng Phi Long Nguyễn Đình Phong Trịnh Bình |
| 2. Decide software architecture | - Define the major software components and interfaces. - Draw core flow diagram, use case diagram, prototype… - Group meeting to review and modify. | Hoàng Phi Long Nguyễn Đình Phong Trịnh Bình |
| 3. Decide Android App GUI | - UX/UI Design for Android Application | Nguyễn Đình Phong Trịnh Bình |
| 4. Design database | - Design database for the system. | Hoàng Phi Long Nguyễn Đình Phong Trịnh Bình |

Table 8: Plain Detail - Design

#### Phrase 3: Implementation

| Task | Description | Author |
| --- | --- | --- |
| 1. Collect temperature, humidity datasets | Program a small embedded program to collect data from sensors | Nguyễn Đình Phong Trịnh Bình |
| 2. Construct hardware system | Build system from hardware components Draw and print PCB board | Hoàng Phi Long Nguyễn Đình Phong Trịnh Bình |
| 3. Implement embedded software system | Develop embedded program to control the system. | Hoàng Phi Long Nguyễn Đình Phong Trịnh Bình |
| 4. Implement Android GUI | Using React Native and Expo to implement Android Application GUI with fake datas | Hoàng Phi Long Nguyễn Đình Phong |
| 5. Build REST API | Using NodeJS & ExpressJS building REST API for Mobile app and the system | Hoàng Phi Long Trịnh Bình |

Table 9: Plain Detail – Implementation

#### Phrase 4: Testing

| Task | Description | Author |
| --- | --- | --- |
| 1. Integration testing | Write test case and testing system. | Hoàng Phi Long Nguyễn Đình Phong Trịnh Bình |
| 2. Alpha testing | Do alpha test with customer. | Hoàng Phi Long Nguyễn Đình Phong Trịnh Bình |

Table 10: Plain Detail –Testing

#### Phrase 5: Maintenance

| Task | Description | Author |
| --- | --- | --- |
| 1. Installation guide | Write installation guide. | Hoàng Phi Long |
| 2. User Manual | Write user manual. | Hoàng Phi Long Nguyễn Đình Phong Trịnh Bình |

Table 11: Plain Detail –Maintenance

# Software & Hardware Requirement Specification

## User Requirement Specification

User is a person who use our device and mobile application. These are functions that user can use:

- Login to mobile application

- Control system to collecting or drying clothes by RF Remote control

- Control system to collecting or drying clothes by button on hardware

- Control system to collecting or drying clothes by android application

- Check information of the system

- Setup and control dryer to dry their clothes when there is a rain

- Manage/edit contact or account information (Name, Address, Mobile phone, Username, Password, …)

## System Requirement Specification

### External Interface Requirement

#### User Interface

The user interface uses English language for mobile application, hardware display interface. General requirement for graphics user interface should be simple, clear, intuitive, and reminiscent. The User interface should design with the following rules:

- User interface is created by using model top-down, left-right design.

- The interface design is an iterate process includes: design, sketching, prototyping, user assessment.

- Some design principles will be taken into consideration:

  - How To Design A Great User Interface – WDD Staff

#### Hardware Interface

Server:

- RAM: 512MB

- CPU: Intel Xeon X5550 @ 2.67GHz

- Disk Storage:

  - Operating System: Minimum 512MB (depends on Operating system)

  - Runtime Environment: 55MB

  - Application server: 60MB

  - Total: 615 MB

Android Phone:

- RAM: Minimum 512MB

- Operating System: Android 4.4 or later

- Network connection: Wi-Fi 802.11 a/b/g/n/ac, 3G, 4G/LTE

- Disk Storage: Minimum 16MB

### System Overview Usecase

#### Hardware System Usecase

Figure 2: Hardware system overview usecase diagram

#### Android Application Usecase

Figure 3: Android application overview usecase diagram

## Hardware Requirement Specification

### Hardware Interface

The hardware interface must have satisfied the following requirements:

- Easy to replace

- Low-cost module

- Easy to implement

Figure 4: System block diagram

Based on project requirement we have choose following hardware components

Conceptual Diagram

Figure 5: Conceptual diagram

Data Dictionary

| Entity Name | Description |
| --- | --- |
| Customer | Contains information of customer who brought our product |
| Product | Contains information about product |
| Model | Contains information about product’s model |
| User | Contains information about account of the system |
| Message | A message queue, contains a message to communicate with hardware system |

Table 12: Data dictionary for conceptual diagram

# Software & Hardware Design Description

## Design Overview

This document describes the technical and user interface design of DCDCS System. It
includes the architectural design, the detailed design of common functions and business
functions and the design of database model.

The architectural design describes the overall architecture of the system and the
architecture of each main component and subsystem.

The detailed design describes static and dynamic structure for each component and
functions. It includes class diagrams, class explanations and sequence diagrams for each
use cases.

The database design describes the relationships between entities and details of each
entity.
Document overview:

- Section 1: Introduction

- Section 2: Gives an overall description of the system architecture design

- Section 3: Gives component diagrams that describe the connection and
integration of the system

- Section 4: Gives the detail design description which includes class diagram,
class explanation, and sequence diagram to details the application functions

- Section 5: Describe a fully attributed ERD

- Section 6: Describe algorithms

## System Architectural Design

Figure 6: System overview architecture

2.1 	API Web Server Architectural Design

Figure 7: API Web server architecture

In API development, the system is developed under MVC architecture style. We choose this architecture for API because of following advantages:

- With MVC architecture, we can separate business code with Controller and View, so we can use the business code in API web server without repeat the code.

- It can eliminate the creation of the singleton and factory classes and well defined interface to business layer

- By separating concerns into 3 distinct pieces, we can perform unit testing easily. Our Presentation layer can be tested free of the Model or Controller, and vice-a-versa

- It supports all aspects of application development, business aspects, persistence aspects, etc., so we can develop a complete application.

This project follows MVC architecture with following components:

- Controller: is the parts of the application that acts like event handler to handles user interaction. Typically, controller reads data from a request and calls appropriate business’s method then selects view to return to user.

- View: The view renders the contents of a model. It gets data from the model and specifies how that data should be presented. It updates data presentation when the model changes. A view also forwards user input to a controller. Depending on the task being performed by the user the model can be looked at from different perspectives.

- Model: Represents the business data and any business logic that govern access to and modification of the data. The model notifies views when it changes and lets the view query the model about its state. It also lets the controller access application functionality encapsulated by the model. Typically, when a change in the model is to be reflected from user, it should be reflected in all the model’s views.

Android Application Architectural Design

Figure 8: Android application internal architecture

In Android application, the system is developed under Flux architecture. We choose this architecture for Android Application because of following advantages:

- Flux is all about controlling the flow inside the app — and making it as simple to understand as possible.

- Easy to implement and understand. Hence it makes source code easier to maintain and reduce time to develop application

- Having supported library (Redux)

- Suitable for React Native codebase

Android Application follows Flux architecture with following components:

- Actions: Helpers that pass data to the Dispatcher. Are simple objects with a type property and some data. For example, an action could be:

{“type”: “IncreaseCount”, “payload”: {“delta”: 1}}

- Dispatcher: Receives these Actions and broadcast payloads to registered callbacks. Acts as a central hub. The dispatcher processes actions (for example, user interactions) and invokes callbacks that the stores have registered with it. The dispatcher isn’t the same as controllers in the MVC pattern — usually the dispatcher does not have much logic inside it and you can reuse the same dispatcher across projects

- Stores: Contain the application’s state and logic. The best abstraction is to think of stores as managing a particular domain of the application. They aren’t the same as models in MVC since models usually try to model single objects, while stores in Flux can store anything. The real work in the application is done in the Stores. The Stores registered to listen in on the actions of the Dispatcher will do accordingly and update the Views.

- Views: are controller-views, also very common in most GUI MVC patterns. They listen for changes from the stores and re-render themselves appropriately. Views can also add new actions to the dispatcher, for example, on user interactions. The view are usually coded in React, but it’s not necessary to use React with Flux.

### Hardware System Architecture

Figure 9: Hardware system architecture

In Embedded Hardware control application, the system is developed under Internet of Things architecture style. We choose this architecture for Embedded Hardware control application because of following advantages:

- Highly scalable and available out of the box due to the nature of each selected component.

- Minimal knowledge required to start.

- It’s scalable and fault tolerant by design.

- Reduces the development and deployment costs and timeframes

The system follows IoT architecture with following components:

- Sensors and Actuators: this part measures a physical quantity such as sound, temperature, moisture etc. and converts it into electrical quantity to make the system understand and act accordingly

- Connectivity (NodeMCU): The received signals are to be uploaded on the network using different communication medium such as Wi-Fi, Bluetooth or BLE, LoPAN etc.

- People and Processes: Networked inputs are then combined into bidirectional system that integrate data, people and processes for better decision making.

Figure 10: Component diagram

Component Diagram

COMPONENT DIAGRAM DICTIONARY: DESCRIBE COMPONENTS

| Component Name | Description |
| --- | --- |
| RF Component | Component to handle RF Remote |
| Rain Sensor Component | Component to handle Rain sensor |
| Keypad Component | Component to handle Keypad |
| NodeMCU Component | Component to handle Wifi, API Request/Response |
| Processing Component | Component to control the system |
| Light Sensor Component | Component to handle Light sensor |
| Dryer Component | Component to handle dryer |
| Display Component | Component to display system’s information |
| DC Motor Component | Component to handle DC motor |
| API Handler | Component to handle API Request/Response on Android |
| Controllers | A group of components that help control android app |
| (View) Login | Login screen |
| (View) Home | Home screen |
| (View) User Profile | User profile screen |
| System Database | Component to handle with database |
| Mongoose | Component to handle request/response and mapping document to Javascript object |
| Controllers | A group of components that help handling API request |
| Express Web Server | A component help build a API server |

Table 13: Component diagram dictionary

## Detailed Description

### Class Diagram

#### API Web Server

Figure 11: API Web Server Class Diagram Part 1

Figure 12: API Web Server Class Diagram Part 1

Figure 13: API Web Server Class Diagram Part 2

Figure 14: API Web Server Class Diagram Part 3

| Class Name | Mapped Column on Conceptual Diagram | Description |
| --- | --- | --- |
| CustomerModel | Customer | Contains customer information |
| ProductModel | Product | Contains product information |
| UserModel | User | Contains user account information |
| ModelModel | Model | Contains model of product information |
| MessageModel | Message | Contains message which used to communicate with hardware system |
| CustomerController | N/A | This class has functions that will handle any request about customer |
| ProductController | N/A | Contains functions that will handle any request about product |
| UserController | N/A | A class with functions that will handle any request about user, login, change password, etc. |
| ModelController | N/A | Contains functions that will handle any request about product model |
| MessageController | N/A | A class has functions that will allow user to publish and get action message |
| Auth | N/A | Authorize request based on access token |
| Router | N/A | A class that listen to request so that the server can call the correct controller |
| Mongoose | N/A | A class help connect and communicate, handle request/response from MongoDB |

Table 14: API Web server class diagram dictionary

Figure 15: Hardware system controller class diagram part 1

Figure 16: Hardware system controller class diagram part 2

Hardware System

| Class Name | Description |
| --- | --- |
| CentralController | The class that receive data from another class and tell SystemController class to control the system correctly |
| SystemController | This class will determine and control system with given action key |
| SwitchHandler | Handler class for limit switch |
| RFHandler | Handler class for limit switch |
| KeypadHandler | Handler class for 4x4 matrix keypad |
| LightSensorHandler | Handler class for light sensor to read light density and determine it is night or day |
| RainSensorHandler | Handler class for rain sensor. |
| WifiHandler | Handle event from NodeMCU that send through I2C Protocol |
| LCDHandler | A class that help print to LCD more easier |
| Wire | External library that help communicate with another device via I2C Protocol |
| DHT | An external library that help reading data from DHT Module |
| BHT1750 | An external library that help reading data from Light Sensor Module |
| DCMotor | This class help controlling dc motor to collect or dry clothes |
| DryerController | This class help controlling dryer fan |
| Action | This is an enum that descriptions the control action of the system |
| SystemStatus | This is an enum that descriptions the status of the system |
| DCDirection | This is an enum that descriptions the status of the dc motor |

Table 15: Hardware controller class diagram dictionary

### Interaction Diagram

#### Sequence Diagrams

##### Control system from android application

Summary: This diagrams show how android application and hardware system can communicate with each other. [ACTION] can be DRY_CLOTHES, COLLECT_CLOTHES, START_DRYER, STOP_DRYER

Figure 17: Control system with android app sequence diagram

##### Update system information

Figure 18: Update system information sequence diagram

Summary: This diagrams show how android application gathers information from hardware system

#### Activity Diagrams

##### Control DC

Figure 19: Control DC activity diagarm

Summary: This diagrams show how user can control the DC

##### Control Dryer

Figure 20: Control Dryer activity diagram

Summary: This diagrams show how user can control the dryer

##### Auto control

Summary: This diagrams show how system itself control.

Figure 21: Auto control activity diagram

## Database Design

### Entity Relational Database (ERD)

Figure 22: Entity Relational Database

### Data Dictionary

| Entity Name | Description |
| --- | --- |
| Customer | Contains information of customer who brought our product |
| Product | Contains information about product |
| Model | Contains information about product’s model |
| User | Contains information about account of the system |
| Messages | A message queue, contains a message to communicate with hardware system |

Table 16: Entity diagram data dictionary

## Algorithms

### System Control

#### Definition

System has many ways to control the system; i.e. RF Remote, Android application, hardware button. From these controllers, they can control many another devices like DC Motor to collect or dry clothes.

#### Define Problem

While using multiple controller at the same time. It causes a collision that leading to the system doesn’t work correctly.

#### Solution

We use one thread and blocking I/O to sequentially reading each controller. Therefore, when we’re handling a single controller. Another controller will be ignored.

#### Pros & Cons

- Pros:

  - No more collisions

  - Easy to control because the system now works on priority of the controller

  - Easy to extends when there are new controller

  - Memory reduced due to using only a single thread

- Cons:

  - An action takes longer time than user to complete (due to the priority)

#### Algorithm Complexity

- Time: O(n) with n is the number of controller

- Space: O(1) because we don’t use any additional spaces

#### Overview Flowchart

Figure 23: System Control overview flowchart

Task Sheet

| No | Product Deliverables | Task | LongHP | PhongND | BinhT | Unit | Size |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Report 1 - Introduction | Project Information | O |  |  |  | 1 |
|  |  | Introduction | O |  |  |  | 1 |
|  |  | Current Situation | O |  |  |  | 1 |
|  |  | Problem Definition | O |  |  |  | 1 |
|  |  | Proposed Solution |  |  |  |  |  |
|  |  | Feature Functions | O |  |  |  | 1 |
|  |  | Advantages and Disadvantages | O |  |  |  | 1 |
|  |  | Functional Requirement | O |  |  |  | 1 |
|  |  | Roles and Responsibility | O |  |  |  | 1 |
|  |  | Conclusion | O |  |  |  | 1 |
| 2 | Report 2 - Software Project Management Plan | Problem Definition |  |  |  |  |  |
|  |  | Name of this Capstone Project |  |  | O |  | 1 |
|  |  | Problem Abstract |  |  | O |  | 1 |
|  |  | Problem Overview |  |  |  |  |  |
|  |  | Current Situation |  | O |  |  | 1 |
|  |  | The Proposed System |  | O |  |  | 1 |
|  |  | Boundaries of the system | O |  |  |  | 1 |
|  |  | Future Plan | O |  |  |  | 1 |
|  |  | Development Environment |  |  |  |  |  |
|  |  | Hardware Development Environment Requirement | O |  |  |  | 1 |
|  |  | Software Development Environment Requirement | O |  |  |  | 1 |
|  |  | Project Organization |  |  |  |  |  |
|  |  | Software Process Model | O |  |  |  | 1 |
|  |  | Roles and Responsibility | O |  |  |  | 1 |
|  |  | Tools and Techniques |  | O |  |  | 1 |
|  |  | Project Management Plan |  |  |  |  |  |
|  |  | Software development life cycle | O |  |  |  | 1 |
|  |  | Phase Detail | O |  |  |  | 1 |
|  |  | Task sheet | O |  |  |  | 1 |
|  |  | All Meeting Minutes |  | O | O |  | 1 |
|  |  | Coding Convention |  |  |  |  | 1 |
|  |  | C++ | O |  |  |  | 1 |
|  |  | Javascript | O |  |  |  | 1 |
|  | Report 3 - Software Requirement Specification | Software & Hardware Requirement Specification |  |  |  |  |  |
|  |  | User Requirement Specification | O |  |  |  | 1 |
|  |  | System Requirement Specification |  |  |  |  |  |
|  |  | External Interface Requirements |  |  |  |  | 1 |
|  |  | User Interface | O |  |  |  | 1 |
|  |  | Hardware Interface | O |  |  |  | 1 |
|  |  | System Overview Use Case |  |  |  |  |  |
|  |  | Hardware System Usecase | O | O |  |  | 3 |
|  |  | Android Application Usecase | O | O |  |  | 3 |
|  |  | List of Use cases |  |  |  |  |  |
|  |  | <Guest> Overview Usecase |  | O |  |  | 1 |
|  |  | <Android User> Overview Usecase |  |  |  |  |  |
|  |  | <Android User> Stop dryer |  | O |  |  | 1 |
|  |  | <Android User> Start dryer |  | O |  |  | 1 |
|  |  | <Android User> Setup dryer timer |  | O |  |  | 1 |
|  |  | <Android User> Control DC to dry clothes |  | O |  |  | 1 |
|  |  | <Android User> Control DC to collect clothes |  | O |  |  | 1 |
|  |  | <Android User> Change controlling device |  | O |  |  | 1 |
|  |  | <Android User> View system information |  | O |  |  | 1 |
|  |  | <Android User> View user information |  | O |  |  | 1 |
|  |  | <Android User> Change user information |  | O |  |  | 1 |
|  |  | <Android User> Change user password |  | O |  |  | 1 |
|  |  | <Android User> Logout |  | O |  |  | 1 |
|  |  | <System User> Overview Usecase |  |  |  |  | 3 |
|  |  | <System User> Turn on power |  | O |  |  |  |
|  |  | <System User> Turn off power |  | O |  |  |  |
|  |  | <System User> Stop dryer |  | O |  |  |  |
|  |  | <System User> Start dryer |  | O |  |  |  |
|  |  | <System User> Set dryer timer |  | O |  |  |  |
|  |  | <System User> View system information |  | O |  |  |  |
|  |  | <System User> Pause DC Motor |  | O |  |  |  |
|  |  | <System User> Control DC to dry clothes |  | O |  |  |  |
|  |  | <System User> Control DC to collect clothes |  | O |  |  |  |
|  |  | <System User> Connect to Wi-Fi |  | O |  |  |  |
|  |  | <System> Overview Usecase |  |  |  |  | 1 |
|  |  | <System> Control DC to collect clothes | O |  |  |  |  |
|  |  | <System> Show information | O |  |  |  |  |
|  |  | <System> Broadcast Wi-Fi | O |  |  |  |  |
|  |  | <System> Send HTTP Request | O |  |  |  |  |
|  |  | <System> Get HTTP Response | O |  |  |  |  |
|  |  | Hardware Requirement Specification |  |  |  |  |  |
|  |  | Hardware Interface |  |  |  |  |  |
|  |  | Rain Sensor |  |  | O |  | 1 |
|  |  | Arduino Mega 2560 R3 |  |  | O |  | 1 |
|  |  | Humidity & Temperature DHT11 |  |  | O |  | 1 |
|  |  | Light Sensor BH1750 |  |  | O |  | 1 |
|  |  | LCD Nokia 5110 |  |  | O |  | 1 |
|  |  | Matrix Keypad (4x4) |  |  | O |  | 1 |
|  |  | Limit Switches |  |  | O |  | 1 |
|  |  | DC Motor GA37 125RPM |  |  | O |  | 1 |
|  |  | Solar Panel |  |  | O |  | 1 |
|  |  | Solar Charge Controller |  |  | O |  | 1 |
|  |  | GTZ5S-E Battery |  |  | O |  | 1 |
|  |  | NodeMCU |  |  | O |  | 1 |
|  |  | Communication Protocol |  |  |  |  |  |
|  |  | I2C Protocol |  |  | O |  | 1 |
|  |  | SPI Protocol |  |  | O |  | 1 |
|  |  | UART Protocol |  |  | O |  | 1 |
|  |  | HTTP Protocol |  |  | O |  | 1 |
|  |  | System Attribute |  |  |  |  |  |
|  |  | Usability | O |  |  |  | 1 |
|  |  | Reliability | O |  |  |  | 1 |
|  |  | Availability | O |  |  |  | 1 |
|  |  | Maintainability | O |  |  |  | 1 |
|  |  | Portability | O |  |  |  | 1 |
|  |  | Performance | O |  |  |  | 1 |
|  |  | Security | O |  |  |  | 1 |
|  |  | Conceptual Diagram | O |  |  |  | 1 |
|  | Report 4 - Software Design Description | Design Overview | O |  |  |  |  |
|  |  | System Architectural Design |  |  |  |  |  |
|  |  | API Web Server Architectural Design | O |  |  |  | 1 |
|  |  | Android Application Architectural Design |  |  |  |  |  |
|  |  | Android Application Overview Architecture | O |  |  |  | 1 |
|  |  | Android Application Internal Architecture | O |  |  |  | 1 |
|  |  | Hardware System Architecture | O |  |  |  | 1 |
|  |  | Component Diagram | O |  | O |  | 1 |
|  |  | Detailed Description |  |  |  |  |  |
|  |  | Class diagram |  |  |  |  |  |
|  |  | API Web Server | O |  |  |  | 2 |
|  |  | Hardware System | O |  |  |  | 2 |
|  |  | Class Diagram Explanation |  |  |  |  |  |
|  |  | UserModel | O |  | O |  | 1 |
|  |  | ModelModel | O |  | O |  | 1 |
|  |  | MessageModel | O |  | O |  | 1 |
|  |  | CustomerModel | O |  | O |  | 1 |
|  |  | ProductModel | O |  | O |  | 1 |
|  |  | Mongoose | O |  | O |  | 1 |
|  |  | UserController | O |  | O |  | 1 |
|  |  | ModelController | O |  | O |  | 1 |
|  |  | MessageController | O |  | O |  | 1 |
|  |  | CustomerController | O |  | O |  | 1 |
|  |  | ProductController | O |  | O |  | 1 |
|  |  | Router | O |  | O |  | 1 |
|  |  | Auth | O |  | O |  | 1 |
|  |  | SystemController | O |  | O |  | 1 |
|  |  | DryerController | O |  | O |  | 1 |
|  |  | WifiHandler | O |  | O |  | 1 |
|  |  | DCMotor | O |  | O |  | 1 |
|  |  | LightSensorHandler | O |  | O |  | 1 |
|  |  | LCDHandler | O |  | O |  | 1 |
|  |  | SwitchHandler | O |  | O |  | 1 |
|  |  | RFHandler | O |  | O |  | 1 |
|  |  | KeypadHandler | O |  | O |  | 1 |
|  |  | RainSensorHandler | O |  | O |  | 1 |
|  |  | CentralController | O |  | O |  | 1 |
|  |  | BHT1750 | O |  | O |  | 1 |
|  |  | Wire | O |  | O |  | 1 |
|  |  | DHT | O |  | O |  | 1 |
|  |  | Interaction Diagram |  |  |  |  |  |
|  |  | Sequence Diagrams | O |  |  |  | 1 |
|  |  | Control system from android application | O |  |  |  | 1 |
|  |  | Update system information | O |  |  |  | 1 |
|  |  | Activity Diagrams |  |  |  |  |  |
|  |  | Control DC | O |  |  |  | 1 |
|  |  | Control Dryer | O |  |  |  | 1 |
|  |  | Auto control | O |  |  |  | 1 |
|  |  | Determine action based on sensors’ data | O |  |  |  | 1 |
|  |  | Determine DC Motor action | O |  |  |  | 1 |
|  |  | Determine dryer action | O |  |  |  | 1 |
|  |  | Interface |  |  |  |  |  |
|  |  | Guest Interface | O |  |  |  | 1 |
|  |  | User Interface |  |  |  |  |  |
|  |  | Home Screen | O |  |  |  | 1 |
|  |  | Control DC Motor | O |  |  |  | 1 |
|  |  | Control Dryer | O |  |  |  | 1 |
|  |  | Select Product | O |  |  |  | 1 |
|  |  | User Profile | O |  |  |  | 1 |
|  |  | Database Design |  |  |  |  |  |
|  |  | Entity Relational Database (ERD) |  |  | O |  | 1 |
|  |  | Data Dictionary |  | O |  |  | 1 |
|  |  | Algorithms |  |  |  |  |  |
|  |  | System Control |  |  |  |  |  |
|  |  | Definition | O |  |  |  | 1 |
|  |  | Define Problem | O |  |  |  | 1 |
|  |  | Solution | O |  |  |  | 1 |
|  |  | Pros & Cons | O |  |  |  | 1 |
|  |  | Algorithm Complexity | O |  |  |  | 1 |
|  |  | Overview Flowchart | O |  |  |  | 1 |
|  | Report 5 - System Implementation and Test | Introduction |  |  |  |  |  |
|  |  | Overview | O |  |  |  | 1 |
|  |  | Test approach | O |  |  |  | 1 |
|  |  | Database Relationship Diagram |  |  |  |  |  |
|  |  | Physical Diagram | O |  |  |  | 1 |
|  |  | Data Dictionary | O |  |  |  | 1 |
|  |  | Performance Measures |  |  |  |  |  |
|  |  | Control System from Android App Performances |  | O |  |  | 1 |
|  |  | Control System from RF Remote/Keypad |  | O |  |  | 1 |
|  |  | Test plan |  |  |  |  |  |
|  |  | Features to be tested |  |  |  |  | 1 |
|  |  | Android Application | O |  |  |  |  |
|  |  | Hardware System | O |  |  |  |  |
|  |  | API Web Server | O |  |  |  |  |
|  |  | Features not to be tested | O |  |  |  | 1 |
|  |  | System Testing Test Case |  |  |  |  |  |
|  |  | State Machine Diagram |  |  |  |  |  |
|  |  | Control DC Motor |  |  | O |  | 1 |
|  |  | Control Dryer |  |  | O |  | 1 |
|  |  | Control System |  |  | O |  | 1 |
|  |  | Android Application Test Case |  |  |  |  |  |
|  |  | Gather System Information |  | O | O |  | 1 |
|  |  | Gather Customer Information |  | O | O |  | 1 |
|  |  | Control DC Motor |  | O | O |  | 1 |
|  |  | Control Dryer |  | O | O |  | 1 |
|  |  | Change Control Device |  | O | O |  | 1 |
|  |  | Chang user/customer information |  | O | O |  | 1 |
|  |  | Hardware System Test Case |  |  |  |  |  |
|  |  | Control from RF Remote/Keypad |  | O | O |  | 1 |
|  |  | Auto control |  | O | O |  | 1 |
|  |  | Connect to Wi-Fi |  | O | O |  | 1 |
|  |  | API Web Server Test Case | O |  |  |  | 1 |
| 6 | Report 6 - Software User's Manual | Installation Guide |  |  |  |  |  |
|  |  | Setup environment at server side |  |  |  |  |  |
|  |  | Hardware Requirements |  |  |  |  |  |
|  |  | Hardware System Requirement | O |  |  |  | 1 |
|  |  | Server Hardware Requirement | O |  |  |  | 1 |
|  |  | Android Hardware Requirement | O |  |  |  | 1 |
|  |  | Software Requirement | O |  |  |  | 1 |
|  |  | API Web Server Deployment Process |  | O |  |  | 1 |
|  |  | Android Application Deployment Process |  | O |  |  | 1 |
|  |  | System Controller Deployment Process |  |  | O |  | 1 |
|  |  | System API Handler Deployment Process |  |  | O |  | 1 |
|  |  | User’s Guide |  |  |  |  |  |
|  |  | Hardware Configuration |  |  |  |  |  |
|  |  | Connect to local Wi-Fi | O |  |  |  | 1 |
|  |  | Android Application |  |  |  |  |  |
|  |  | Control DC |  |  |  |  |  |
|  |  | Dryer Clothes | O |  |  |  | 1 |
|  |  | Collect Clothes | O |  |  |  | 1 |
|  |  | Control Dryer |  |  |  |  |  |
|  |  | Setup timer and start dryer |  | O |  |  | 1 |
|  |  | Stop the dryer |  | O |  |  | 1 |
|  |  | Change Control Device |  |  | O |  | 1 |
|  |  | Change User Information |  |  | O |  | 1 |

Table 17: Task sheet

# Appendix

- Flux Architecture: https://facebook.github.io/flux/

- How Expo Works: https://docs.expo.io/versions/latest/workflow/how-expo-works

- React Native Mechanism Explanation: https://wetalkit.xyz/react-native-what-it-is-and-how-it-works-e2182d008f5e

- Bit Twiddling Hacks: https://graphics.stanford.edu/~seander/bithacks.html

- Understanding Node.js & Express.js: https://medium.com/@LindaVivah/the-beginners-guide-understanding-node-js-express-js-fundamentals-e15493462be1

- Understand Express: https://evanhahn.com/understanding-express/

- Visual Diagram Guide: https://www.visual-paradigm.com/guide/

- The 4 stages of an IoT architecture: https://techbeacon.com/4-stages-iot-architecture