|  | MINISTRY OF EDUCATION AND TRAINING |
| --- | --- |

# FPT UNIVERSITY

Capstone Project Document

DESIGN AND CONSTRUCTION SUN DRYING WET CLOTHES SYSTEM

| Group 2 |  |
| --- | --- |
| Group members | Hoàng Phi Long – SE62021 Nguyễn Đình Phong – SE Trịnh Bình – SE |
| Supervisor | Nguyễn Đức Lợi |
| Ext. Supervisor | N/A |
| Capstone Project code | DCDCS |

-Ho Chi Minh City, June 26th 2018

This page is intentionally left blank

# Table of Contents

Table of Contents	2

List of Tables	5

List of Figures	8

Definitions, Acronyms and Abbreviations	10

A.	Introduction	11

1.	Project Information	11

2.	Introduction	11

3.	Current Situation	11

4.	Problem Definition	12

5.	Proposed Solution	12

5.1	Feature Functions	12

5.2	Advantages and Disadvantages	12

6.	Functional Requirements	13

7.	Role and Responsibility	14

8.	Conclusion	14

B.	Software Project Management Plan	15

1.	Problem Definition	15

1.1	Name of this Capstone project	15

1.2	Problem Abstract	15

1.3	Project Overview	15

2.	Project Organization	18

2.1	Software Process Model	18

2.2	Roles and Responsibility	19

2.3	Tools and Techniques	20

3.	Project Management Plan	21

3.1	System Development Life-cycle	21

3.2	Plan Detail	23

4.	Coding Convention	26

C.	Software & Hardware Requirement Specification	28

1.	User Requirement Specification	28

2.	System Requirement Specification	28

2.1	External Interface Requirement	28

2.2	System Overview Usecase	30

2.3	Usecase Details	32

3.	Hardware Requirement Specification	33

3.1	Hardware Interface	33

3.2	Communication Protocol	46

4.	System Attributes	49

4.1	Usability	49

4.2	Reliability	49

4.3	Availability	49

4.4	Maintainability	49

4.5	Portability	49

4.6	Performance	50

4.7	Security	50

5.	Conceptual Diagram	51

D.	Software & Hardware Design Description	52

1.	Design Overview	52

2.	System Architectural Design	53

2.1 	API Web Server Architectural Design	54

2.2	Android Application Architectural Design	56

2.3	Hardware System Architecture	60

3.	Component Diagram	62

4.	Detailed Description	64

4.1	Class Diagram	64

4.2	Class Diagram Explanation	71

4.3	Interaction Diagram	88

5.	Interface	96

5.1	Guest Interface	96

5.2	User Interface	98

6.	Database Design	106

6.1	Entity Relational Database (ERD)	106

6.2	Data Dictionary	106

7.	Algorithms	107

7.1	System Control	107

E.	System Implementation & Testing	109

F.	Software & Hardware User’s Manual	110

G.	Acknowledgement	111

H.	Appendix	112

# List of Tables

Table 1: General Roles and Responsibilities of Member	14

Table 2: Hardware development environment requirement for DCDCS System	17

Table 3: Software development environment requirement for DCDCS System	18

Table 4: Roles and responsibilities	20

Table 5: Tools and techniques	20

Table 6: Project task planning	22

Table 7: Plain Detail - Requirement Analysis	23

Table 8: Plain Detail - Design	24

Table 9: Plain Detail – Implementation	25

Table 10: Plain Detail –Testing	25

Table 11: Plain Detail –Maintenance	25

Table 12: Specifications for rain sensor	34

Table 13: Specifications for Arduino mega 2560	36

Table 14: Specifications for DHT11	37

Table 15: Specifications for BH1750	37

Table 16: Specifications for LCD5110	38

Table 17: Specifications for 4x4 matrix keypad	39

Table 18: Specifications for limit switch	40

Table 19: Specifications for DC Motor GA37	41

Table 20: Specifications for solar panel	42

Table 21: Specifications for charge controller	43

Table 22: Specifications for GTZ5S-E battery	44

Table 23: Specifications for NodeMCU	45

Table 24: Data dictionary for conceptual diagram	51

Table 25: Component diagram dictionary	63

Table 26: API Web server class diagram dictionary	67

Table 27: Hardware controller class diagram dictionary	70

Table 28: Attribute dictionary for UserModel	71

Table 29: Method dictionary for UserModel	71

Table 30: Attribute dictionary for ModelModel	72

Table 31: Method dictionary for ModelModel	72

Table 32: Attribute dictionary for MessageModel	73

Table 33: Method dictionary for MessageModel	73

Table 34: Attribute dictionary for CustomerModel	74

Table 35: Method dictionary for CustomerModel	74

Table 36: Attribute dictionary for ProductModel	75

Table 37: Method dictionary for ProductModel	75

Table 38: Method dictionary for Mongoose	76

Table 39: Method dictionary for UserController	76

Table 40: Method dictionary for ModelController	77

Table 41: Method dictionary for MessageController	77

Table 42: Method dictionary for CustomerController	78

Table 43: Method dictionary for ProductController	78

Table 44: Method dictionary for Router	79

Table 45: Method dictionary for Auth	79

Table 46: Attribute dictionary for SystemController	80

Table 47: Method dictionary for SystemController	80

Table 48: Attribute dictionary for DryerController	81

Table 49: Method dictionary for DryerController	81

Table 50: Method dictionary for WifiHandler	81

Table 51: Attribute dictionary for DCMotor	82

Table 52: Method dictionary for DCMotor	82

Table 53: Method dictionary for LightSensorHandler	82

Table 54: Attribute dictionary for LCDHandler	83

Table 55: Method dictionary for LCDHandler	83

Table 56: Attribute dictionary for SwitchHandler	83

Table 57: Method dictionary for SwitchHandler	83

Table 58: Attribute dictionary for RFHandler	84

Table 59: Method dictionary for RFHandler	84

Table 60: Attribute dictionary for KeypadHandler	84

Table 61: Method dictionary for KeypadHandler	84

Table 62: Attribute dictionary for RainSensorHandler	85

Table 63: Method dictionary for RainSensorHandler	85

Table 64: Method dictionary for Central Controller	85

Table 65: Attribute dictionary for BHT1750	86

Table 66: Method dictionary for BHT1750	86

Table 67: Attribute dictionary for Wire	86

Table 68: Method dictionary for Wire	86

Table 69: Attribute dictionary for DHT	87

Table 70: Method dictionary for DHT	87

Table 71: <Guess> Login screen fields table	97

Table 72: <Guess> Login screen buttons/hyperlinks table	97

Table 73: <User> Home screen fields table	99

Table 74: <User> Home screen buttons/hyperlinks	99

Table 75: <User> DC Motor control screen fields	100

Table 76: <User> DC Motor control screen buttons/hyperlinks	101

Table 77: <User> Setup and control dryer screen fields	102

Table 78: <User> Setup and control dryer screen buttons/hyperlinks	102

Table 79: <User> Select controlling product screen fields	103

Table 80: <User> Select controlling product screen buttons/hyperlinks	104

Table 81: <User> User's profile screen fields	105

Table 82: <User> User's profile buttons/hyperlinks	105

Table 83: Entity diagram data dictionary	106

# List of Figures

Figure 1: Waterfall methodology	18

Figure 2: Hardware system overview usecase diagram	30

Figure 3: Android application overview usecase diagram	31

Figure 4: System block diagram	33

Figure 5: Rain sensor	34

Figure 6: Arduino Mega 2650 R3	35

Figure 7: Module DHT11	36

Figure 8: Module Light sensor BH1750	37

Figure 9: Module LCD5110	38

Figure 10: 4x4 Matrix keypad	39

Figure 11: Limit switch	40

Figure 12: DC Motor GA37	41

Figure 13: 10W Solar Panel	42

Figure 14: Solar Charge Controller	43

Figure 15: GTZ5S-E Battery	44

Figure 16: NodeMCU v1.0	45

Figure 17: I2C Protocol	46

Figure 18: SPI Protocol	47

Figure 19: UART Protocol	47

Figure 20: Arduino communicates with server through HTTP Protocol	48

Figure 21: Conceptual diagram	51

Figure 22: System overview architecture	53

Figure 23: API Web server architecture	54

Figure 24: Android application overview architecture	56

Figure 25: Android application internal architecture	58

Figure 26: Hardware system architecture	60

Figure 27: Component diagram	62

Figure 28: API Web Server Class Diagram Part 1	64

Figure 29: API Web Server Class Diagram Part 1	64

Figure 30: API Web Server Class Diagram Part 2	65

Figure 31: API Web Server Class Diagram Part 3	66

Figure 32: Hardware system controller class diagram part 1	68

Figure 33: Hardware system controller class diagram part 2	69

Figure 34: Control system with android app sequence diagram	88

Figure 35: Update system information sequence diagram	89

Figure 36: Control DC activity diagarm	90

Figure 37: Control Dryer activity diagram	91

Figure 38: Auto control activity diagram	92

Figure 39: Determine action based on sensors’ data activity diagram	93

Figure 40: Determine dc motor action activity diagram	94

Figure 41: Determine dryer action activity diagram	95

Figure 42: <Guess> Login screen	96

Figure 43: <User> Home screen	98

Figure 44: <User> Control DC Motor screen	100

Figure 45: <User> Setup and control dryer screen	101

Figure 46: <User> Select controlling product screen	103

Figure 47: <User> User's profile screen	104

Figure 48: Entity Relational Database	106

Figure 49: System Control overview flowchart	108

# Definitions, Acronyms and Abbreviations

| Name | Definition |
| --- | --- |
| DCDCS | Design and Construction sun Drying wet Clothes System |
| RF | Radio frequency |
| HTTP | Hypertext transfer protocol |
| I2C | Inter-Integrated Circuit |
| UART | Universal asynchronous receiver-transmitter |
| DIY | Do it yourself |
| REST | Representational state transfer |
| API | Application programming interface |
| GPIO | General-purpose input/output |
| I/O | Input/Output |

# Introduction

## Project Information

- Project name: DESIGN AND CONSTRUCTION SUN DRYING WET CLOTHES SYSTEM

- Project Code: DCDCS

- Product Type: Embedded Device, Android Application, API Web Server

- Start Date: 14/06/2018

- End Date: 31/08/2018

## Introduction

In this document, we introduce a solution for automatic clothes drying system. We build a system, which use rain sensor to detect rain, ESP8266 for communication between Android application and embedded device.

This document also describes our working process in 4 months includes our perspective in the system, component designs and detailed core workflows. We hope the system will help resolve some aspects of the problem that the current face recognition systems are facing today.

## Current Situation

Vietnam is a rainy country, with 6 months of sunshine and 6 months of rainy. Vietnamese people prefer drying their clothes under sunshine, wind over using clothes dryer or another dryer machines. When the rainy season comes, Vietnamese people tend to worry about their clothes at home being wet by rains. There are a few solutions to solve this problem as known as “Smart Clothesline Rigs”. This device really expensive and not really that smart. With 13.000.000 VND, you can have controllable system with UV light, build-in dryer and remote control within 30 meters. However, this system is not really solve the core problem: Automatically collecting clothes. Therefore, we come to this solution, helping Vietnamese people not to worry about their clothes during rainy season.

## Problem Definition

With systems currently available on market

Advantage of their system:

- UV disinfection

- Built-in dryer

- Strong structure can lift up to 25kg of clothes

- Below are disadvantages of current situation:

- Current systems have high production costs

- Hard to extend

- Control manually when the electricity is down

- Cannot automatically collecting clothes when rain

## Proposed Solution

Our proposed solution is designing and construction automatic clothes drying system called DCDCS to solve missing feature of current “Smart Clothesline Rigs”. Our system will help users automatically collect clothes when it is a rain. It is much cheaper, easy to install and mobile and extendable.

DCDCS system includes a mobile app and an embedded device with following functions:

### Feature Functions

- Mobile App:

  - Control the system through wireless

  - Check weather information

  - Check system status

- Embedded Device:

  - Check system status

  - Control system through hard buttons

### Advantages and Disadvantages

- Advantages:

  - Low costs

  - Can detect rain very fast

  - Can control with mobile app

  - Use solar energy and have battery to storage unused energy

- Disadvantages:

  - Cannot detect whether the clothes is dry or not

  - Cannot detect whether rain is over or not

## Functional Requirements

Functional requirements of the system are listed as below:

- Embedded system component:

  - RESTful API communication through wireless

- Use Arduino Mega 2560 as a central circuit unit

- Show information about the system

- Time

- Temperature

- Humidity

- Control dryer

- Control clothesline

- Power supply component:

- Power supply operates for the entire system

- Distributed voltage 5V and 12V

- Auto charging

- Storing energy

- User component:

- Control the system from Android application through wireless

- Turning on/off build-in dryer

- Set timer for dryer

- Control the clothesline

- Check system status and weather

- Edit user information

- Name

- Address

- Mobile phone

- Etc

- Mobile Application component:

- Communicate with system through wireless and by REST API

- Show information about the system

- Time

- Temperature

- Humidity

- Weather (Rain or not)

- System status

## Role and Responsibility

| No | Full name | Role | Position | Contact |
| --- | --- | --- | --- | --- |
| 1 | Nguyễn Đức Lợi | Project Manager | Supervisor | loinnd@fpt.edu.vn |
| 2 | Hoàng Phi Long | Developer | Leader | longhpse62021@fpt.edu.vn |
| 3 | Nguyễn Đình Phong | Developer | Member | phongndse@fpt.edu.vn |
| 4 | Trịnh Bình | Developer | Member | binhtse@fpt.edu.vn |

Table 1: General Roles and Responsibilities of Member

## Conclusion

- Research to determine and implement the appropriate MCU for the Central Control Unit and other nodes

- Design and implement integrate PCB board.

- Research and implement NoSQL Database, RESTful API, Mobile Application.

- C, C++ embedded into Arduino.

- Use software in design PCB, Schematic such as OrCAD, Proteus

- Communication technique: TCP, HTTP

# Software Project Management Plan

## Problem Definition

### Name of this Capstone project

- Official name: Design and construction sun drying wet clothes system

- Vietnamese name: Thiết kế và xây dựng hệ thống phơi đồ tự động

- Abbreviation: DCDCS

### Problem Abstract

Vietnamese people work all day long. They spend time at evening and night to do their housework. One of the housework that is washing clothes then drying them. However, Vietnam is a rainy country. During rain season, everybody very worry about their drying clothes at home getting wet.

### Project Overview

#### Current Situation

Below are the problems encountered in the project:

- Hard to improve the system: Our system is a very simple system. However, to improve the system is a hard mission. Our system currently cannot detect when the clothes are dry, when the rain is stopped for auto collecting clothes or continue drying wet clothes. To do so, it requires mathematics model called Hidden Markov Models. However, due to the lack of knowledge in statistics and linear algebra; we are currently unable to implement this model.

- Lack of knowledge in telecommunication: While using ESP8266, we found out that there are some interferences during transmission. Without telecommunication, we do have hard time to detect the problem.

#### The Purposed System

According to the technology researches, we found that the simple rain sensor and ESP8266 Wi-Fi module is capable in solving the problem. We can use rain sensor detect raining and ESP8266 for wireless communication.

We assign task responsibility vertically to make sure if any member in this project fail in our team, harm would be minimized for the project.

We also build a mobile application for real-time demonstration.

#### The Boundaries of the system

Our system provides these functions:

- Automatically control clothesline when there is a rain or at night.

- Dryer system so that user can dry their clothes on rainy days

- Control system via RF Remote control

- Control system via Button on the system

- Check system status and control system via Mobile application

#### Future plans

- Implement Hidden Markov Models (HMM) for rain forecasting

- Implement the system can determine when the rain has stopped using HMM

- Build a website for user to check their account information and control the system along with mobile application

- Build a system that can detect whenever the clothes is dry or wet

#### Development Environments

##### Hardware Development Environment Requirement

For CCU clothes drying system

| Component | Hardware |
| --- | --- |
| Mainboard | Arduino Mega 2560 |
| Communication | Wire and cable |
| Devices | - Module real-time clock DS1307 - Rain sensor - Humidity and Temperature sensor DHT11 - Light sensor BH1750 - DC Motor - Nokia 5110 LCD - 4x4 Matrix keypad - Limit switches - Solar Panel - Battery - … |
| Power source | 5V – 12V |
| Android Device | Any android mobile phone has 3G/4G or Wi-Fi connection |

Table 2: Hardware development environment requirement for DCDCS System

##### Software Development Environment Requirement

| Software | Name / Version |
| --- | --- |
| Operating System | Windows 7 or above |
| Environment/Run-time | Adruino Mega 2560 NodeJS |
| Modeling tool | Draw.io for UML Proteus 8 for PCB Board |
| IDE | Visual Studio Code Arduino IDE |
| DBMS | MongoDB |
| Source control | Git-scm and Github |
| Communication tools | Facebook Messenger Gmail |

Table 3: Software development environment requirement for DCDCS System

## Project Organization

### Software Process Model

This project is developed using modified waterfall model. We apply modified waterfall model because it suitable with current situation in our team. We choose this model because of the following reasons:

- This project is 4-months long due to the FPT University Capstone Project timeline, which can be consider a short project.

- Based on researches and current clarified face recognition system, the requirements of this project are stable, clear, fixed and well-understood by all team members.

Figure 1: Waterfall methodology

The Modified Waterfall Model involves verification and validation between the phases, so any deviations can be corrected immediately, providing the customer satisfaction, so this is preferred.

### Roles and Responsibility

| No | Fullname | Role | Responsibilities |
| --- | --- | --- | --- |
| 1 | Nguyễn Đức Lợi | Supervisor, Project Manager | - Specify user requirement - Advisor for ideas and solutions - Give out techniques and business analysis support |
| 2 | Hoàng Phi Long | Team leader, developer, tester | - Managing process - Dividing tasks for team member - Create test plan - Clarifying requirements - Coding - Testing - Verify document - Managing budget - Database design |
| 3 | Nguyễn Đình Phong | Team member, developer, tester | - Create test plan - Database design - Clarifying requirements - Prepare document - Coding - Testing - GUI Design |
| 4 | Trịnh Bình | Team member, developer, tester | - Create test plan - GUI Design - Database design - Clarifying requirements - Prepare document - Coding - Testing |

Table 4: Roles and responsibilities

### Tools and Techniques

| Tools |  |
| --- | --- |
| Developing tools | Visual Studio Code Arduino IDE |
| Database system management | MongoDB |
| Source Control | Git-scm and Github |
| Models and Diagrams tool | Draw.io |
| Techniques |  |
| Embedded System | C/C++ , Arduino SDK |
| API Web Server System | ExpressJS & NodeJS |
| Mobile Application | React Native, Javascript |

Table 5: Tools and techniques

## Project Management Plan

### System Development Life-cycle

Below are all the major tasks that need to be performed sequentially during the development of the system.

| Phase | Description | Deliverables | Resource needed | Dependencies and Constraints | Risks |
| --- | --- | --- | --- | --- | --- |
| Requirement Analysis | - Identify and clarify main functions. - Prepare task plan. - Research mechanics of collecting clothes system - Research solar energy circuit | - Report No. 1 Introduction. - Project Management Plan - Task sheet - Prototypes | 14 man-days | N/A | - Missing requirement. - Unclear project’s scope. - Lack of member share of understand. |
| Design | - Identify hardware and software requirements. - Decide software architecture. - GUI design using top-down break down. - Design database. | - Report No. 2 Software Project Management Plan. - Report No. 3 Software Requirement Specification. - Report No. 4 Software Design Description. | 20 man-days | Depend on “Requirement Analysis”. | - Misunderstood or unclear system’s requirement. - Lack of practical experience leading to unreasonable design. |
| Implementation | - Collect temperature, humidity datasets. - Build hardware system - Implement embedded software system - Implement Android GUI. - Build REST API | - Demonstration application. - Report No.5 System Implementation & Test. | 50 man-days | Depend on “Design”. | - Lack of practical experience and knowledge. - Human mistake. - Broken hardwares due to wrong implementation - Interference signal while ESP8266 communicate with Http Protocal |
| Testing | - Prepare test plan and test case. - Test all functions and results. | Report No.5 System Implementation & Test. | 20 man-days | Depend on “Implementation”. | - Lack of experience. - Not enough time for performing test. - Missing bugs. - Human resource. |
| Maintenance | - Deploy the system. - Create the user’s manuals. | Report No.6 Software User’s Manual. | 10 man-days | Depend on “Testing”. | - Lack of experience and knowledge. - Human mistake. - User’s manual may be difficult for user to understand and confuse. |

Table 6: Project task planning

### Plan Detail

#### Phrase 1: Requirement Analysis

| Task | Description | Author |
| --- | --- | --- |
| 1. Research mechanics of collecting clothes system | - Research on current systems, their strengths and weakness. | Hoàng Phi Long Nguyễn Đình Phong |
| 1. Research solar energy | - Research on current systems, their strengths and weakness. - Research how to convert solar to electricity and charge into batter | Nguyễn Đình Phong Trịnh Bình |
| 3. Identify and clarify main functions | Define main and needed functions the system must include. | Hoàng Phi Long Nguyễn Đình Phong Trịnh Bình |
| 4. Create system introduction | Complete Introduction Report. | Hoàng Phi Long |
| 5. Software Project Management Plan | Prepare Project Management Plan. | Hoàng Phi Long |
| 6. Prototype | Build a prototype of system and mobile application. | Nguyễn Đình Phong Trịnh Bình |
| 7. SRS | Create SRS document. | Hoàng Phi Long Nguyễn Đình Phong Trịnh Bình |

Table 7: Plain Detail - Requirement Analysis

#### Phrase 2: Design

| Task | Description | Author |
| --- | --- | --- |
| 1. Identify hardware and software detail design | Find out the suitable hardware and software for the system, as well as its minimum and recommended requirements. | Hoàng Phi Long Nguyễn Đình Phong Trịnh Bình |
| 2. Decide software architecture | - Define the major software components and interfaces. - Draw core flow diagram, use case diagram, prototype… - Group meeting to review and modify. | Hoàng Phi Long Nguyễn Đình Phong Trịnh Bình |
| 3. Decide Android App GUI | - UX/UI Design for Android Application | Nguyễn Đình Phong Trịnh Bình |
| 4. Design database | - Design database for the system. | Hoàng Phi Long Nguyễn Đình Phong Trịnh Bình |

Table 8: Plain Detail - Design

#### Phrase 3: Implementation

| Task | Description | Author |
| --- | --- | --- |
| 1. Collect temperature, humidity datasets | Program a small embedded program to collect data from sensors | Nguyễn Đình Phong Trịnh Bình |
| 2. Construct hardware system | Build system from hardware components Draw and print PCB board | Hoàng Phi Long Nguyễn Đình Phong Trịnh Bình |
| 3. Implement embedded software system | Develop embedded program to control the system. | Hoàng Phi Long Nguyễn Đình Phong Trịnh Bình |
| 4. Implement Android GUI | Using React Native and Expo to implement Android Application GUI with fake datas | Hoàng Phi Long Nguyễn Đình Phong |
| 5. Build REST API | Using NodeJS & ExpressJS building REST API for Mobile app and the system | Hoàng Phi Long Trịnh Bình |

Table 9: Plain Detail – Implementation

#### Phrase 4: Testing

| Task | Description | Author |
| --- | --- | --- |
| 1. Integration testing | Write test case and testing system. | Hoàng Phi Long Nguyễn Đình Phong Trịnh Bình |
| 2. Alpha testing | Do alpha test with customer. | Hoàng Phi Long Nguyễn Đình Phong Trịnh Bình |

Table 10: Plain Detail –Testing

#### Phrase 5: Maintenance

| Task | Description | Author |
| --- | --- | --- |
| 1. Installation guide | Write installation guide. | Hoàng Phi Long |
| 2. User Manual | Write user manual. | Hoàng Phi Long Nguyễn Đình Phong Trịnh Bình |

Table 11: Plain Detail –Maintenance

# Software & Hardware Requirement Specification

## User Requirement Specification

User is a person who use our device and mobile application. These are functions that user can use:

- Login to mobile application

- Control system to collecting or drying clothes by RF Remote control

- Control system to collecting or drying clothes by button on hardware

- Control system to collecting or drying clothes by android application

- Check information of the system

- Setup and control dryer to dry their clothes when there is a rain

- Manage/edit contact or account information (Name, Address, Mobile phone, Username, Password, …)

## System Requirement Specification

### External Interface Requirement

#### User Interface

The user interface uses English language for mobile application, hardware display interface. General requirement for graphics user interface should be simple, clear, intuitive, and reminiscent. The User interface should design with the following rules:

- User interface is created by using model top-down, left-right design.

- The interface design is an iterate process includes: design, sketching, prototyping, user assessment.

- Some design principles will be taken into consideration:

  - How To Design A Great User Interface – WDD Staff

#### Hardware Interface

Server:

- RAM: 512MB

- CPU: Intel Xeon X5550 @ 2.67GHz

- Disk Storage:

  - Operating System: Minimum 512MB (depends on Operating system)

  - Runtime Environment: 55MB

  - Application server: 60MB

  - Total: 615 MB

Android Phone:

- RAM: Minimum 512MB

- Operating System: Android 4.4 or later

- Network connection: Wi-Fi 802.11 a/b/g/n/ac, 3G, 4G/LTE

- Disk Storage: Minimum 16MB

### System Overview Usecase

Figure 2: Hardware system overview usecase diagram

Hardware System Usecase

#### Android Application Usecase

Figure 3: Android application overview usecase diagram

## Hardware Requirement Specification

### Hardware Interface

The hardware interface must have satisfied the following requirements:

- Easy to replace

- Low-cost module

- Easy to implement

Based on project requirement we have choose following hardware components

Figure 4: System block diagram

## System Attributes

### Usability

- Users can install the system easily.

- Users can learn how to use the system quickly.

- The system is divided into several components to facilitate the installation into maintenance

### Reliability

- Control signal from mobile application controls correctly the system 95% accuracy

- The system must detect the rain correctly with 99% accuracy

- Received data from temperature and humidity sensors with 90% accuracy

- Response the action from RF Remote with 90% accuracy

- Response the action from buttons with 95% accuracy

### Availability

- The mechanical component requires electrical system to work well.

- System must reply within 3 seconds from Wi-Fi command and less than 1 second from RF Remote or buttons onboard.

- System must work in cloudy day and night

- Wi-Fi broadcast range is maximum 50 meters

### Maintainability

- The system is subdivided into modules so it is easy to replace.

- Hardware components are easy to find in the market or DIY.

### Portability

- Easy to construct

- Mobility

- User can use the mobile application on devices running Android 4.0 or later

### Performance

- Fast signal response, less than 3000ms.

- Fast system response, less than 1000ms

- System can handle 1000 requests at one time

### Security

- Each role of user has a specific permission to interact with the system.

- System always checks for authorization and authentication before doing anything.

- Only Administrator can grant permission to other roles.

- System owner can only control their own systems

- Only system owner can only control their systems

# Software & Hardware Design Description

## Design Overview

This document describes the technical and user interface design of DCDCS System. It
includes the architectural design, the detailed design of common functions and business
functions and the design of database model.

The architectural design describes the overall architecture of the system and the
architecture of each main component and subsystem.

The detailed design describes static and dynamic structure for each component and
functions. It includes class diagrams, class explanations and sequence diagrams for each
use cases.

The database design describes the relationships between entities and details of each
entity.
Document overview:

- Section 1: Introduction

- Section 2: Gives an overall description of the system architecture design

- Section 3: Gives component diagrams that describe the connection and
integration of the system

- Section 4: Gives the detail design description which includes class diagram,
class explanation, and sequence diagram to details the application functions

- Section 5: Describe screens design

- Section 6: Describe a fully attributed ERD

- Section 7: Describe algorithms

Figure 22: System overview architecture

System Architectural Design

Figure 27: Component diagram

Component Diagram

COMPONENT DIAGRAM DICTIONARY: DESCRIBE COMPONENTS

| Component Name | Description |
| --- | --- |
| RF Component | Component to handle RF Remote |
| Rain Sensor Component | Component to handle Rain sensor |
| Keypad Component | Component to handle Keypad |
| NodeMCU Component | Component to handle Wifi, API Request/Response |
| Processing Component | Component to control the system |
| Light Sensor Component | Component to handle Light sensor |
| Dryer Component | Component to handle dryer |
| Display Component | Component to display system’s information |
| DC Motor Component | Component to handle DC motor |
| API Handler | Component to handle API Request/Response on Android |
| Controllers | A group of components that help control android app |
| (View) Login | Login screen |
| (View) Home | Home screen |
| (View) User Profile | User profile screen |
| System Database | Component to handle with database |
| Mongoose | Component to handle request/response and mapping document to Javascript object |
| Controllers | A group of components that help handling API request |
| Express Web Server | A component help build a API server |

Table 25: Component diagram dictionary

## Detailed Description

### Class Diagram

#### API Web Server

Figure 28: API Web Server Class Diagram Part 1

Figure 29: API Web Server Class Diagram Part 1

Figure 30: API Web Server Class Diagram Part 2

Figure 31: API Web Server Class Diagram Part 3

| Class Name | Mapped Column on Conceptual Diagram | Description |
| --- | --- | --- |
| CustomerModel | Customer | Contains customer information |
| ProductModel | Product | Contains product information |
| UserModel | User | Contains user account information |
| ModelModel | Model | Contains model of product information |
| MessageModel | Message | Contains message which used to communicate with hardware system |
| CustomerController | N/A | This class has functions that will handle any request about customer |
| ProductController | N/A | Contains functions that will handle any request about product |
| UserController | N/A | A class with functions that will handle any request about user, login, change password, etc. |
| ModelController | N/A | Contains functions that will handle any request about product model |
| MessageController | N/A | A class has functions that will allow user to publish and get action message |
| Auth | N/A | Authorize request based on access token |
| Router | N/A | A class that listen to request so that the server can call the correct controller |
| Mongoose | N/A | A class help connect and communicate, handle request/response from MongoDB |

Table 26: API Web server class diagram dictionary

Figure 32: Hardware system controller class diagram part 1

Hardware System

Figure 33: Hardware system controller class diagram part 2

| Class Name | Description |
| --- | --- |
| CentralController | The class that receive data from another class and tell SystemController class to control the system correctly |
| SystemController | This class will determine and control system with given action key |
| SwitchHandler | Handler class for limit switch |
| RFHandler | Handler class for limit switch |
| KeypadHandler | Handler class for 4x4 matrix keypad |
| LightSensorHandler | Handler class for light sensor to read light density and determine it is night or day |
| RainSensorHandler | Handler class for rain sensor. |
| WifiHandler | Handle event from NodeMCU that send through I2C Protocol |
| LCDHandler | A class that help print to LCD more easier |
| Wire | External library that help communicate with another device via I2C Protocol |
| DHT | An external library that help reading data from DHT Module |
| BHT1750 | An external library that help reading data from Light Sensor Module |
| DCMotor | This class help controlling dc motor to collect or dry clothes |
| DryerController | This class help controlling dryer fan |
| Action | This is an enum that descriptions the control action of the system |
| SystemStatus | This is an enum that descriptions the status of the system |
| DCDirection | This is an enum that descriptions the status of the dc motor |

Table 27: Hardware controller class diagram dictionary

## Database Design

Figure 48: Entity Relational Database

Entity Relational Database (ERD)

### Data Dictionary

| Entity Name | Description |
| --- | --- |
| Customer | Contains information of customer who brought our product |
| Product | Contains information about product |
| Model | Contains information about product’s model |
| User | Contains information about account of the system |
| Messages | A message queue, contains a message to communicate with hardware system |

Table 83: Entity diagram data dictionary

## Algorithms

### System Control

#### Definition

System has many ways to control the system; i.e. RF Remote, Android application, hardware button. From these controllers, they can control many another devices like DC Motor to collect or dry clothes.

#### Define Problem

While using multiple controller at the same time. It causes a collision that leading to the system doesn’t work correctly.

#### Solution

We use one thread and blocking I/O to sequentially reading each controller. Therefore, when we’re handling a single controller. Another controller will be ignored.

#### Pros & Cons

- Pros:

  - No more collisions

  - Easy to control because the system now works on priority of the controller

  - Easy to extends when there are new controller

  - Memory reduced due to using only a single thread

- Cons:

  - An action takes longer time than user to complete (due to the priority)

#### Algorithm Complexity

- Time: O(n) with n is the number of controller

- Space: O(1) because we don’t use any additional spaces

#### Overview Flowchart

Figure 49: System Control overview flowchart

# Task Sheet

# Appendix

- Flux Architecture: https://facebook.github.io/flux/

- Bit Twiddling Hacks: https://graphics.stanford.edu/~seander/bithacks.html